Level 3 NVQ Diploma Installing Electrotechnical Systems and Equipment

Understanding the requirements for terminating and connecting

UNIT QELTK3/ 005  Sample questions

Name: ……………………………………………….………………………………..………		Date: ………………………………………………………..

Q1. 	BS7671 requires all connections to be readily accessible because:

- It makes terminating easier

- It makes terminating quicker

- It allows for inspecting, testing and maintenance

- It is more cost effective

Q2. 	Which one of the following types of connection must remain accessible:

- A joint designed to be buried in the ground

- A connection to a ceiling-rose

- A joint made by soldering

- A connection between a cold tail and the heating element of a floor heating system

Q3. 	A loose live electrical connection at a terminal could result in:

- Excess current flowing

- Excess heat which could damage insulation

- The operation of a circuit breaker

- A corrosive effect at the terminal

Q4. 	When carrying out a continuity test on a circuit conductor, which of the following measurements would indicate a good connection:

- 0.01 Ω

- >99.9 Ω

- > 1 MΩ

- 9.99 Ω

Q5. 	A cable that must be sealed at each end to prevent the ingress of moisture is:

- PVC sheathed/ PVC insulated multi-core

- PVC conduit/ PVC insulated single

- PVC sheathes SWA/ PVC insulated multi-core

- Mineral insulated

Q6. 	Non-sheathed cables must be enclosed to provide which of the following:

- Protection against over-heating

- Protection against over-voltage

- Protection against mechanical damage

- Protection against the ingress of liquids

Q7. 	A BW type gland used to terminate SWA cable has:

- No seal

- A seal between inner and outer sheaths

- A seal between conductor insulation and inner sheath

- A seal between outer sheath and gland body

Q8. 	Which one of the following cable designations indicate a cable that would be used in data wiring:

- M.I.C.C.

- PVC/ SWA

- TWISTED PAIR

- FP200

Q9. 	A CW type gland used to terminate SWA cable has:

- No seal

- A seal between inner and outer sheaths

- A seal between conductor insulation and inner sheath

- A seal between outer sheath and gland body

Q10. 	A cable that provides a fire-resisting property is:

- FP200

- PVC/ SWA

- PVC/ PVC

- Cat 5e

Q11. 	Which of the following is not required to be considered in relation to the connection between conductors, or between conductors and other equipment:

- The material of the conductor and its insulation

- The number and shape of the wires forming the conductor

- The number of conductors to be connected together

- The type of over-current device protecting the circuit

Q12. 	Which of the following types of electrical connections need not be accessible:

- A joint made in a junction box to the appropriate BS(EN) standard

- A joint made by soldering

- A joint made at a terminal block to the appropriate BS(EN) standard

- A joint made using a ceiling rose to the appropriate BS(EN) standard

Q13. 	Where a high level of heat might be a problem, for example a heater connection block, the terminal block should be made of:

- nylon

- steel

- porcelain

- pvc

Q14. 	A loose or poorly made electrical joint is referred to as a:

- low resistance connection

- high resistance connection

- low capacitance connection

- high capacitance connection

Q15. 	BS7671 states the colour codes for live conductors at the interface between old and new cables of a 3 phase a.c. supply as:

- L1		Old = Red;   	New = Brown

L2		Old = Yellow;  	New = Black

L3		Old = Blue;  	New = Grey

N		Old = Black;  	New = Blue

- L1		Old = Red;   	New = Brown

L2		Old = Yellow;  	New = Grey

L3		Old = Blue;  	New = Blue

N		Old = Black;  	New = Black

- L1		Old = Red;   	New = Brown

L2		Old = Yellow;  	New = Black

L3		Old = Black;  	New = Grey

N		Old = Blue;  	New = Blue

- L1		Old = Red;   	New = Brown

L2		Old = Yellow;  	New = Grey

L3		Old = Blue;  	New = Black

N		Old = Black;  	New = Blue

Q16. 	In a 3 phase power circuit the Line conductors may now be identified by:

- The colours Red, Yellow and Blue at the terminals

- The colours Brown, Black and Blue at the terminals

- Using one of the permitted Line colours for all three conductors

- Any colour if marked L1, L2 and L3 at the terminals

Q17. 	Which section of BS7671 refers to the electrical durability of ELECTRICAL CONNECTIONS?

- Part 5

- Part 2

- Section 526

- Regulation 526.3

Q18. 	BS EN 60309 socket outlets are available in ratings that include:

- 16A, 32A and 63A

- 13A, 32A and 63A

- 3A, 5A and 13A

- 10A, 16A, 20A and 32A

Q19. 	To fit a 20mm SWA BW type gland to a socket outlet supplied with a 25mm female thread, the adaptor required is referred to as:

- A 25 to 20mm female adaptor

- A 25 to 20mm male adaptor

- A 25mm male to 20mm female adaptor

- A 25mm female to 20mm male adaptor

Q20. 	If used as the sole means of support, which of the following cable supports must not be used to support FP cable?

- PVC mini-trunking

- Stainless steel cable ties

- PVC coated steel P-clips

- Steel conduit

Q21. 	What is the minimum size CPC for a BS1363 radial socket circuit wired in singles in steel trunking, protected by a 32amp type B circuit breaker?

- 1.5mm2

- 2.5mm2

- 4.0mm2

- 6.0mm2

Q22. 	The total number of socket outlets spurred directly from a ring circuit (non-fused):

- is unlimited

- must not exceed the total directly connected to the ring

- is one

- is unlimited to a floor area not exceeding 100m2